WEATHERPROOF MUSIC HORN 
LOUDSPEAKER

## MSH30/T

### Architects & Engineers Specification

### The unit shall be the PENTON model MSH30/T or equal and approved. The horn enclosure shall be constructed of high impact ABS with UV inhibitors and non-perishable gaskets to avoid the ingress of water or dust to a minimum IP66. The unit will be mounted via a hipped stainless steel “U” bracket and fixings. The speaker assembly shall comprise of a high-density phenolic resin diaphragm housed in a flared oblong chamber 420x 260x 287.5mm (W x H x D).

A 30 watt/100 volt line, factory fitted transformer shall provide 3dB power taps. Cable entry shall be via a single cable entry on a removable rear cover.

The loudspeaker shall have a controlled dispersion. The cones shall be high compliance units with a smooth extended frequency response.

### Rated power 	30w

### Tappings 100v line	30/15/7.5/3.75w
Effective frequency range               	70 ~16.5Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	99dB                                    
Dispersion at 1k/2kHz                                          	70/120 (Horizontal)
                                                                                	110/110 (Vertical) Dispersion
Axial Q Factor at 1k/2kHz                                    	8.5/6.5

### Colour                                                                        Grey

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org

WEATHERPROOF MUSIC HORN 
LOUDSPEAKER

## MSH30/TC

### Architects & Engineers Specification

### The unit shall be the PENTON model MSH30/TC or equal and approved. The horn enclosure shall be constructed of high impact ABS with UV inhibitors and non- perishable gaskets to avoid the ingress of water or dust to a minimum IP66. The unit will be mounted via a hipped stainless steel “U” bracket and fixings. The speaker assembly shall comprise of a high-density phenolic resin diaphragm housed in a flared oblong chamber 420x 260x 287.5mm (W x H x D).

A 30 watt/100 volt line, factory fitted transformer shall provide 3dB power taps.

Cable entry shall be via a single 16mm cable entry on a removable rear cover.

The loudspeaker shall have a controlled dispersion. The cones shall be high compliance units with a smooth extended frequency response.

The MSH30/TC will be suitable for voice alarm applications and will be supplied with ceramic terminals to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚. A 4-core fire rated cable tail should be added to ensure full compliance or the addition of a second cable entry to allow “loop in, loop out” termination.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

### Rated power 	30w

### Tappings 100v line	30/15/7.5/3.75w
Effective frequency range               	70 ~16.5Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	99dB                                    
Dispersion at 1k/2kHz                               	70/120 (Horizontal)
                                                                                	110/110 (Vertical) Dispersion
Axial Q Factor at 1k/2kHz                                    	8.5/6.5

### Colour                                                                        Grey

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org