MOULDED CABINET

LOUDSPEAKER

PBC10/TCOAX

# Architects & Engineers Specification

The unit shall be the PENTON model PBC10/TCOAX or equal and approved. The enclosure shall be ported with an open back and constructed of high impact ABS plastic, treated with UV inhibitors. The bowed grille will be manufactured from mild steel construction with an epoxy coated finish. The speaker assembly shall comprise of a co-axial, 160mm diameter bass/mid-range treated coned loudspeaker and 25mm Mylar dome tweeter complete with neodymium magnet and shall be fitted with a 10watt / 100 volt line factory fitted transformer with the power taps clearly marked on the assembly.

The co-axial loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

### Rated power 	10w

### Tappings 100v line	10/5/2.5/1.25w
Effective frequency range (BS6840 part 5)              	80 ~20Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	89dB                                    
Dispersion at 1k/2kHz                                                 180/120
Axial Q Factor at 1k/2kHz                                    	3.7/5.9

### Colour                                                                        White

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org

MOULDED CABINET

LOUDSPEAKER

PBC10/TCOAXC

# Architects & Engineers Specification

The unit shall be the PENTON model PBC10/TCOAXC or equal and approved. The enclosure shall be ported with an open back and constructed of high impact ABS plastic, treated with UV inhibitors. The bowed grille will be manufactured from mild steel construction with an epoxy coated finish. The speaker assembly shall comprise of a co-axial, 160mm diameter bass/mid-range treated coned loudspeaker and 25mm Mylar dome tweeter complete with neodymium magnet and shall be fitted with a 10watt / 100 volt line factory fitted transformer with the power taps clearly marked on the assembly.

The co-axial loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

The PBC10/TCOAXC will be suitable for voice alarm applications and will be supplied with ceramic terminals to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

### Rated power 	10w

### Tappings 100v line	10/5/2.5/1.25w
Effective frequency range	80 ~20kHz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	89dB                                    
Dispersion at 1k/2kHz                                                 180/120
Axial Q Factor at 1k/2kHz                                    	3.7/5.9

### Colour                                                                        White

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org