WEATHERPROOF 
HORN LOUDSPEAKERS

## PH10/T

### Architects & Engineers Specification

The unit shall be the PENTON model PH10/T or equal and approved. The horn enclosure shall be constructed of high impact ABS with UV inhibitors and non-perishable gaskets to prevent the ingress of water or dust and be rated to a minimum of IP66. The unit will be mounted via a stainless steel “U” bracket and fixings. The speaker assembly shall comprise of a high-density phenolic resin diaphragm housed in a flared chamber, 200mm long x 138mm diameter.

A 10 watt/100 volt line, factory fitted transformer shall provide 3dB power taps, power selection via multi-core cable.

The loudspeaker shall have a controlled dispersion. The cones shall be high compliance units with a smooth extended frequency response.

### Rated power 	10w

### Tappings 100v line	10/5/2.5/1.25w
Effective frequency range               	400 ~8Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	97dB                                    
Dispersion at 1k/2kHz                                                 180/110
Axial Q Factor at 1k/2kHz                                    	2.8/6.6

### Colour                                                                        Grey

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org

WEATHERPROOF 
HORN LOUDSPEAKERS

## PH10/TC

### Architects & Engineers Specification

The unit shall be the PENTON model PH10/TC or equal and approved. The horn enclosure shall be constructed of high impact ABS with UV inhibitors and non- perishable gaskets to prevent the ingress of water or dust and be rated to a minimum of IP66. The unit will be mounted via a stainless steel “U” bracket and fixings. The speaker assembly shall comprise of a high-density phenolic resin diaphragm housed in a flared chamber, 200mm long x 138mm diameter.

A 10watt / 100 volt line, factory fitted transformer shall provide 3dB power taps

Cable entry will be via a single gland located at the rear of the cabinet on a removable rear cover.

The loudspeaker shall have a controlled dispersion. The cones shall be high compliance units with a smooth extended frequency response.

The PH10/TC will be suitable for voice alarm applications and will be supplied with ceramic terminals to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚. A 4-core fire rated cable tail should be added to ensure full compliance or the addition of a second cable entry to allow “loop in, loop out” termination.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

### Rated power 	10w

### Tappings 100v line	10/5/2.5/1.25w
Effective frequency range               	400 ~8Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	97dB                                    
Dispersion at 1k/2kHz                                                 180/110
Axial Q Factor at 1k/2kHz                                    	2.8/6.6

### Colour                                                                        Grey

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org