###### VANDALPROOF BI-DIRECTIONAL

METAL CABINET LOUDSPEAKER

SENTRY6/BDTC

# Architects & Engineers Specification

This unit shall be the PENTON model SENTRY6/BDTC or equal and approved. The loudspeaker cabinet shall be a two-part epoxy coated steel fabrication with two inner metal mesh grilles covering the drive units. The speaker assembly shall comprise of 2 x 80mm diameter dual coned midrange loudspeakers and a 6watt / 100 volt line factory fitted transformer with the power taps clearly marked on the assembly.

Cable entry will be via 2 x 20mm cable entries located at the top of the cabinet and via concealed universal mounting holes (for standard UK electrical junction boxes) at the rear.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

The SENTRY6/BDTC will be suitable for voice alarm applications and will be supplied with ceramic terminals to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

### Rated power 	6w

### Tappings 100v line	6/3/1.5/0.75
Effective frequency range 	170 ~10Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	88dB                                    
Dispersion at 1k/2kHz                                                 90/89
Axial Q Factor at 1k/2kHz                                    	0.6/0.7

### Colour                                                                        White

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org