VANDALPROOF ROUND
 METAL CABINET LOUDSPEAKER

SENTRY6/RTC

# Architects & Engineers Specification

This unit shall be the PENTON model SENTRYT6/RTC or equal and approved. The round loudspeaker cabinet shall be constructed in cast aluminium with a separate steel fronted chassis and grille epoxy powder coated to match. The speaker assembly shall comprise of a 140mm diameter dual coned loudspeaker and shall be fitted with a 6watt / 100volt line factory fitted transformer with the power taps clearly marked on the assembly.

Cable entry will be via 2 x 20mm cable entries located at the top of the cabinet and via concealed universal mounting holes (for standard UK electrical junction boxes) at the rear.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

The SENTRY6/RTC will be suitable for voice alarm applications and will be supplied with ceramic terminals to take 2 x 2.5mm² conductors and a thermal fuse rated at 150˚.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

### Rated power 	6w

### Tappings 100v line	6/3/1.5/0.75/0.25w
Effective frequency range               	250 ~18Khz

### SPL @ 1w/m (test signal 100Hz~10kHz)               	90dB                                    
Dispersion at 1k/2kHz                                                 140/110
Axial Q Factor at 1k/2kHz                                    	4.6/6.7

### Colour                                                                        White

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org