VANDALPROOF SQUARE
 METAL CABINET LOUDSPEAKER

WITH LED STROBE

SENTRY6/STCL

# Architects & Engineers Specification

SPEAKER

This unit shall be the PENTON model SENTRYT6/STCL or equal and approved. The loudspeaker cabinet shall be constructed in cast aluminium with a separate steel fronted chassis and grille all epoxy powder coated to match. The speaker assembly shall comprise of a 140mm diameter dual coned loudspeaker and shall be fitted with a 6 watt/100 volt line factory fitted transformer with the power taps clearly marked on the assembly.

The loudspeaker shall have a wide-angle dispersion. The cone shall be a damped, high compliance type with a smooth extended frequency response.

Cable entry will be via 2 x 20mm cable entries located at the top of the cabinet or via concealed universal mounting holes (for standard UK electrical junction boxes) at the rear.

The SENTRY6/STCL will be suitable for voice alarm applications and will be supplied with ceramic terminals able to take up to 2 x 2.5mm² conductors per terminal and a thermal fuse rated at 150˚.

This loudspeaker must be compliant to BS5839 part 8 & EN60849.

BEACON

The SENTRYT6/STCL shall be fitted with a high intensity, low profile beacon mounted in the centre of the speaker grill for maximum visibility. The beacon will be fitted with a RED LED and a clear lens.

### Rated power                                                        6w

### Tappings 100v line                                               6/3/1.5/0.75/0.25w
Effective frequency range                                  180 ~18 KHz

### SPL @ 1w/m (test signal 100Hz~10 kHz)              93dB                                    
Dispersion at 1 k/2kHz                                         170/120
Axial Q Factor at 1 k/2kHz                                  4.8/6.3

### Colour                                                                   White

### Voltage / current LED strobe                              24VDC Polarized, 6.3mA (0.15w)
Flash rate of strobe per minute                          55 – 60

### Intensity of Strobe in Lux @ 75mm                      57

All units to be tested in accordance with BSEN60268-5

Penton UK Ltd, Unit 2, Teville Industrials, Dominion Way, Worthing, West Sussex BN14 8NW

Tel: 01903 215315 Fax: 01903 215415 E-Mail: sales@pentonuk.co.uk Web: www.penton.org